Research Paper Summary

Title: UAV/UGV Autonomous Cooperation: UAV Assists UGV to climb a cliff by attaching a tether

Author(s): Takahiro Miki, Petr Khrapchenkov, Koichi Hori

Date: 2019-05

Link to paper: https://github.com/albud187/Thesis_Literature_Review/blob/main/finished_reading/UAV_UGV_Autonomous_Cooperation_UAV_assists_UGV_to_climb_a_cliff_by_attaching_a_tether.pdf

Section 1 - Overall Idea

- UAV tethered to UGV. UAV acts as a sensor but can also be used as an anchor to assist with climbing over certain obstacles.

- Autonomous framework for UAV-UGV collaboration.

- UAV uses 3D voxel mapping and obstacle avoidance planning. UGV uses voxel map and generates elevation map.

Section 2 - Methodology

- UAV tethered to UGV. UAV detects anchoring point and has position estimation, trajectory following, obstacle avoidance and 3D mapping. Visual Inertial Odometry (VIO) for relative pose from reference frame. Model predictive control for higher flight controls and PID for motor speed.

- Sensor fusion with KF (VIO + laser) and EKF (height filtered odo and IMU).

- UGV equipped with winch. UGV path plans basedon traversability analysis from UAV’s sensor measurement data. Dead reckoning with Odometry and IMU. Pure pursuit algorithm for position control. A* algorithm for path planning, optimal path on 2D grid.

- Anchor point detection based on peakness, quantified by elevation variance around cells on the generated map.

- Both systems use ROS.

Section 3 - Applications

- Persistent long range sensor mounted on an armored reconnaissance vehicle, with ability to detach and operate autonomously / be replaced. This can supplement a mast and other sensors on the vehicle.

Section 4 - Future Development

- Modular configurations - different UAV types that can be mounted and swapped for another on a UGV or a truck.

Section 5 - Questions

nil

Section 6 - Anything Else

nil