Research Paper Summary

Title: Robots that Sync and Swarm: A proof of Concept in ROS 2

Author(s): Agata Barcis, Michal Barcis, Christian Bettstetter

Date: 2019-08

Link to paper: https://github.com/albud187/Thesis_Literature_Review/blob/main/finished_reading/Robots_that_Sync_and_Swarm_A_Proof_of_Concept_in_ROS_2.pdf

Section 1 - Overall Idea

- Unified mathematical model for synchronisation and swarming. Each entity (“swarmalator”) coordinates its phase and location with other entities such that these attributes are mutually coupled.

- Multiple space-time patterns, implementation in ROS 2.

- This is a to control pose of multiple robots using a decentralized local approach.

Section 2 - Methodology

- Movement governed by attraction and repulsion functions (input is position difference between 2 entities) as well as phase difference. Position dependent synchronization governed by phase attraction (input is position difference) and dependency of on proximity of the 2 entitiies.

- Orientations can be aligned with neighbouring entities, governed by orientation difference and spatial proximity.

- Balboa robot chosen due to computational power of raspberry pi, self-balancing behaviour and good state estimation capacility.

- Swarmalatorbots acquire their position in global reference frame, indoors a motion capture system was used. Robots acquire its own position from the system to update its own state. Swarmalatorbots communicate with each other to exchange state info using wifi.

Section 4 - Future Development

- This paper seems to present a fairly sophisticated method to control the formation of multiple robots. Though I don’t think it distinguishes which robot should go where.

- If there were different types of robots, then perhaps a certain type must always be in a certain spot in the formation relative to the rest of the robots. For example, a logistic support robot with 4 defender robots. The carrier robot should always be in the middle surrounded by the 4 defender robots.

Section 5 - Questions

- What is meant by “phase”? I don’t think it was clearly defined in this paper.

Section 6 - Anything Else

- Note that each swarmalator oscillates around given position, even with static patterns. Communication delay has huge impact on pattern stationarity if this oscillation is present.