Research Paper Summary

Title: Group Synchronization in Coordination Tasks via Network Control Methods

Author(s): Sidney N. Givigi, Kleber M. Cabral, Peter T. Jardine

Date: 2020-07

Link to paper: https://github.com/albud187/Thesis_Literature_Review/blob/main/finished_reading/Group_Synchroniziation_Network_Control.pdf

Section 1 - Overall Idea

- Coordination among robots and between robots and humans using network control methods.

- Robot needs to observe environment and take action according to the behaviour of other agents, be it other robots or humans.

- Multiple other agents present, virtual agent interaction with real agents. Synchronization between humans and robots.

- Mirror game - synchronization between humans and robots in oscillatory tasks - all players try to synchronize their motion. Agents try to match motion of neighbours.

Section 2 - Methodology

- Graph formulation - network of agents. Virtual agent model - haken-kelso-bunz oscillator. Oscillatory tasks / mirror game. Performance evaluated by relative phase of each agent or position error wrt mean group position.

- Information received via position measurement from agent to agent. States of all nodes converge to a common value.

- Networked control - agent dynamics wrt neighbours - control signal from coupling strength and adjancey matrix. State change function of state, control signal and time.

- Pinning control - local feedback controllers on small fraction of network nodes (pins), remaining nodes influenced by their connection with the pins. Controller objective - all nodes track even nodes not being directly controlled.

- Combine mirror game + pinning control. Agents synchronize themselves to pins

Section 3 - Applications

- It can be implemented in situations where nonlinear control with a virtual model is useful. For example, matching performance of a physical robot with a virtual robot - matching performance from a flight simulator to a real aircraft / UAV.

Section 4 - Future Development

- Implementation for complex cooperative tasks such as assembly of structures.

Section 5 - Questions

- What is phase and what does it mean to be “synchronized”?

- Is this somewhat a form of model reference control? The virtual agent being the model and the system syncrhonizing with the virtual agent’s actions?

Section 6 - Anything Else