Research Paper Summary

Title: Flocking for Heterogenous Robot Swarms: A Military Convoy Scenario

Author(s): Christopher J.R McCook, Joel M. Esposito

Date: 2007-03

Link to paper: https://github.com/albud187/Thesis_Literature_Review/blob/main/finished_reading/Flocking_for_Heterogeneous_Robot_Swarms_A_Military_Convoy_Scenario.pdf

Section 1 - Overall Idea

- Flocking control algorithm is used to control a swarm of heterogenous robots in a military convoy unit with supply vehicles, defenders and attackers

- The flocking controller contains separation, cohesion and velocity matching terms.

- Convoy should be able to drive itself to destination while avoiding obstacles and collision, responding to threats, while maintaining cohesion. Must also be robust and able to function despite loss of individual or multiple units.

- Emergent swarm behaviour with high level sophistication constructed from low level behaviours

Section 2 - Methodology

- Swarm is modeled as 2nd order system with position and velocity of each robot in the system. 2 Part control input, using artificial potential field specific to robot, and using universal velocity matching term for swarm cohesion + alignment.

- 3 types of robot: supply unit, defender and attacker, each with their own control laws.

- Collision avoidance controller is function of robot’s distance from any other robot. This control law creates a “repulsive force” and is used for defender and supply units for collision avoidance.

- Potential function for convoy goal is standard parabolic attractor function.

- Obstacle avoidance function grows to infinity as distance from robot to obstacle approaches zero, up to a cutoff, then it is equal to 0 at or farther than cutoff distance.

- Enemy units modelled as large obstacles for supply robots and modelled as localized goals for defender robots.

Section 3 - Applications

- A simpler application of multi-agent robot delivery system could make use of this system. There could be different robots meant to carry different packages, each of them would have to avoid obstacles and collision with each other.

- This delivery system could be internal to a warehouse or a construction site to help carry construction materials or tools. It would work in more predictable environment. It would need significant development to work in a less predictable environment such as military combat operations.

Section 4 - Future Development

- I think it needs significantly more work or significant reduction in scope before any convoy application, military or otherwise. In a military context, how does a defender distinguish what is an attacker and what isn’t an attacker? What about threats from asymmetric warfare?

- Incorporating data and sensor fusion from a flying UAV. The data from the ground robots with the UAV can provide a better view of the situation and more optimal solutions.

- Developing a method for units to share information of the known work space, both with and without the UAV.

Section 5 - Questions

- How was de-centralized control achieved exactly? I think it was just giving each robot specific rules to follow. Not sure if this is sufficient autonomy for a convoy application.

- How can we maintain robustness and distrubted command with a UAV? For example, the system with a UAV can be considered to be operating in “optimal condition” as it has more information. If the UAV goes down, the system must still operated in a “degraded condition.” How do we mitigate risk / performance loss should the UAV go down?

Section 6 - Anything Else

# nil